On the EBC-855 BIOS Flashing Issue

The correct method of flashing a new BIOS is:

1. Flash the BIOS using phlash16 with the arguments "/mode=3" and "/C".

2. After phlash16 completes, remove "J16".

3. Reboot the ebc855, hit F2 to enter Setup.

4. Replace jumper "J16".

5. Exit Setup, save settings to CMOS and EEPROM.

Invoking phlash16 with the "/C" argument will cause the contents of CMOS to be cleared, and the "/mode=3" argument will cause the entire image (including the DMI string space) to be flashed. The contents of the EEPROM are unaffected. Since the EBC-855 has the EEPROM BIOS settings backup feature, if the CMOS contents are invalid - determined by checksumming - the settings will be loaded from the EEPROM (if valid - again by checksumming, removing the jumper "J16" will cause the contents to be read as blank and the checksumming to fail)). If the both the CMOS and the EEPROM contents are invalid the BIOS will load it's factory default (built-in) settings.Different BIOS versions' settings may differ in either quantity and/or position. A specific BIOS and it's settings are matched, as is the copy of the settings in the EEPROM. Attempting to use settings - whether from the CMOS or the EEPROM - from a different BIOS version can cause undesired behavior, including failure to boot.  Since we specified "/C" on the phlash16 command line, on the subsequent boot the CMOS contents will fail the checksumming, and with "J16" removed, the BIOS will load it's factory default settings.

NOTE: If the EBC-855 BIOS is being flashed remotely, removing and re-installing "J16" may be undesirable. In this case the contents of the EEPROM can be invalidated prior to flashing by using the utility "855tool". 855tool is a menu driven multi-purpose DOS executable and is overkill for this application, however it will effect the desired result.